| Grading description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date: 17th Jan 1997 | Rev No-date: 2-16/09/99 |  |  | Section: QC |  | Number: 3 |
| Products |  | Species | Grade |  | Description |  |
| Michigan Engineered Superior Solid |  | Red Oak & White Oak | Prime |  | Combination of Clear and Select and better Grades. Excellent appearance Natural colour variation, limited character marks. Occasional small live pin knots allowed. Limited sapwood |  |
|  |  |  | Country |  | Combination of No1 Com & No2A Com. Variegated appearance. Natural colour variation Unlimited Sapwood allowed. Sound knots and dead knots (filled) allowed. Excessive large dead knots are not allowed. |  |
|  |  |  | Country + |  | Combination of Prime and Country grades. |  |
|  |  | Hard Maple | Prime |  | Combination of Clear and Select and better Grades. Excellent appearance Natural colour variation, limited character marks. Occasional small live pin knots allowed. Occasional small mineral streaks allowed. Sticker stain not considered a defect. |  |
|  |  |  | Country |  | Combination of No1 Com & No2A Com. Variegated appearance. All colour variation Flags, mineral streaks allowed.. Sound knots and dead knots (filled) allowed. Excessive large dead knots are not allowed. Sticker stain is not considered a defect. |  |
|  |  |  | Country + |  | Combination of Prime and Country grades. |  |
|  |  | American Black Walnut | Prime |  | Combination of Clear and Select and better Grades. Excellent appearance. Natural colour variation, limited character marks. Occasional small live pin knots allowed. Limited sapwood. |  |
|  |  |  | Country |  | Combination of No1 Com & No2A Com. Variegated appearance. Natural colour variation. Unlimited sapwood. Sound knots and dead knots (filled) allowed. Excessive large dead knots are not allowed. |  |
|  |  |  | Country + |  | Combination of Prime and Country grades. |  |